How To Update Document Camera Firmware

Preparation for downloading SOC firmware

1. Prepare SD card (SDHC_4G~32G) to save SOC firmware and format the SD card to FAT32.

2. Save the "ELPDC21_V***_AS.bin" in the SD card.

Downloading SOC firmware

1. Connect the AC cable to the ELPDC21 and insert the SD card where SOC
firmware has been saved, into the ELPDC21.

2. Press [Power] button on ELPDC21 while pressing the [Focus] and [Freeze] button on it.
When upgrade of the firmware is begun, [Record] LED blinks blue.

3. When the firmware upgrad is completed, all LED are turned off.
Note: If the upgrade fails, the [Record] LED flashes in 0.25 seconds cycle.
Please reconnected the AC cable, and execute the upgrade again.

4. - Disconnect the SD card from the ELPDC21.
- Next, connect a computer cable (or a HDMI cable) to a projector (or a monitor) and the ELPDC21.
- Disconnect the SD card from ELPDC21, and restart it.

5. When the upgrade is completed properly, Version # of the upgraded FW is
 displayed next to Version 1 in Info menu of the ELPDC21.